# Mitre Saw Induction V1.2

Operator Name: Date:

Machine identification and setting The individual is able to correctly identify, describe and set/adjust the following: a. Mitre angle controls and detents. b. Cross cut controls. c. Start and stop controls.. d. Compound controls and fence adjustment.. Machine set up and operation No loose clothing. Long hair tied back.. Hands 150mm from blade at all times.. The maximum size sections that can be safely cut. Correct adjustment of rear fence during compound cuts.. Correct placement of workpiece during operation.. Safe operation during plunge cross and compound cuts. Use of multiple shallow cuts.. Correctly use of safety devices. Machine is started at the top and and must be returned to the top position after cut.. Always cut from front to rear (away from user) when cross cutting.. Never to cross arms when cutting.. When cutting uneven materials use clamps to steady workpiece.. Check machine zero before use.. No lone working. Tooling The individual is able to: a. Identify dull and defective saw blades and teeth.